Learning the Architecture of NVIDIA GPUs:

We will look at the Microarchitecture Maxwell found in the GTX 900 family.

The GPU architecture is built around a scalable array of streaming multiprocessor or SMs. GPU hardware parallelism is achieved through by replication of this architectural building block. In this diagram, the SMs the image at the left.

We can count 16 such streaming multiprocessors(SMs) in this GPU. Those streaming multiprocessors have access to device inbuilt global memory and L2 cache, memory controls and cache.

The key components of streaming multiprocessor are listed in this diagram. (We’ll use not the SM of the Fermi microarchitecture for this example.)

Modern SM units are somewhat different than this, but we can clearly identify basic components from this diagram. We have:

- CUDA cores: like a CPU core and it will execute instructions fetch into that core.

- Shared Memory & L1 Cache: facilitate faster memory access

- Registers: limited amount of on-chip memory to store on the fly values for execution

- Load and store Units: perform memory load and store requests

- Warp Schedulers: determine which warp or which set of threads are going to execute next

- Special Function Units

We have to especially careful how we use resources like registers, shared memory and L1 cache, as those are very limited resources on SM.

Warp schedulers will take resource consumption factor when it deciding which warp is going to execute next.

Modern SM are more complicated.

Here this diagram shows you SM for the Volta microarchitecture.

All these green squares are cores. But these SM have different types of cores like:

- Tensor cores (TC)

- Integer operation execution cores (Int)

- FP32 cores (optimized to execute 32-bit floating-point operations faster)

- FP64 cores (optimized to execute 64-bit floating-point operations faster)

Computer architectures can be classified into four categories according to Flynn’s taxonomy (a classification of Computer Architectures):

SISD (Single Instructions Single Data):  Represents sequential computers which exploits no parallelism in either the instruction or data streams. Here a single control unit will fetch a single instruction stream from memory. The control unit then generates appropriate controls signal to direct the signal processing element to operate on single data stream that is on operation at a time.

SIMD (Single Instruction Multiple Data): Represents the organization of single computer containing control unit, processor unit and memory unit. The instructions are going to execute sequentially. This can be achieved by pipelining or multiple function units.

MIMD (Multiple Instruction Single Data):  Here multiple instruction operates on one data stream. This is an uncommon architecture which is generally used for fault tolerance (the ability of a system to continue operating without interruption when one or more of its components fail). Here heterogeneous systems will operate on same data stream and must agree on the results. Examples: Space Shuttle Flight Control Computers.

MIMD (Multiple Instruction Multiple Data): Represents multiple autonomous processors simultaneously executing different instruction on different data. MIMD architectures includes multicore super scalar processors and it distribute its system using either one shared memory space or distributed memory space.

Cuda follows a version of computer architecture called SIMT.

SIMT is an extension of SIMD model with multithreading (single instruction is going to run on multiple threads)

In CUDA, thread block is going to execute in single streaming multiprocessor. Multiple thread blocks can be executed simultaneously on same streaming multiprocessor depending on the resource limitation of that SM.

But one thread block CANNOT be executing in multiple SMs. If the device cannot run single thread block in one SM (EX: If the needed shared memory count for a particular block is greater than the available shared memory in SM) then an error will return for that kernel launch. When a block is schedule to execute on SM, it is executed so a single instruction is run by multiple threads.

In previous sections, we arrange a set of threads to set of blocks on a grid. That's what we able to observe from software point of view.

On the hardware level, we can map these execution as shown in the diagram.

- The grid of the kernel launch is going to map to device.

- Thread blocks in this grid can be divide across SM in the device.

- A single thread in a thread block is going to be executed by one single core.

- And since we have multiple cores in a single SM, the same instruction can execute using multiple threads in the SM (to adhere the SIMT convention).

DIFFERENT CUDA MICROARCHITECTURES CONTAIN DIFFERENT AND ENHANCED CAPABILITIES.